Name:	__________________

Date:	__________________

Period:	__________________

Circle the best answer.

- Which of the following statements is correct?

  - A street elbow has one male thread and one female thread.

  - A bushing has two female threads.

  - A reducer has one male thread and one female thread.

  - A 90 degree elbow has two male threads.

- The most common way to identify size of pipe is by:

  - Inside diameter

  - Outside diameter

  - Wall thickness

  - Pressure tolerance

- The proper sequence for assembling PVC pipe is:

  - Prime, cut, glue, assemble

  - Cut, prime, assemble, glue

  - Cut, prime, glue, assemble

  - Cut, sand, glue, assemble

- Soft 50-50 solder is a combination of:

  - Tin and lead

  - Tin and zinc

  - Gold and zinc

  - Silver and zinc

- What type of threads are found on steel pipe fittings?

  - Straight pipe threads

  - Straight machine threads

  - Hose threads

  - Tapered pipe threads

- A valve used for water supply systems to prevent backflow of water is the:

  - Gate valve

  - Globe valve

  - Check valve

  - Cock valve

- Which pipe fitting would be used to fasten the ends of pipes where neither pipe can be turned?

  - Coupling

  - Reducer

  - Bushing

  - Union

- Soldering flux is not essential when sweat soldering copper pipe.

  - True

  - False

- When threading steel pipe, the pipe should extend beyond the die at least 1/2 inch.

  - True

  - False

- The standard length for steel pipe is:

  - 10 feet

  - 12 feet

  - 15 feet

  - 20 feet

  - 21 feet

- Bushing pipe fittings have:

  - One male thread and one female thread

  - Two male threads

  - Two female threads

- During the soldering process, the solder is distributed between surfaces of the bare metals by:

  - Capillary action

  - Caterpillar action

  - Pressure

  - Osmosis

- Soldering is a process of:

  - Fusion

  - Fission

  - Adhesion

  - Cohesion

- Copper tubing can be joined by which of the following methods?

  - Flare fittings

  - Sweating

  - Solvent welding

  - Both a and b

  - Both a and c

- Steel pipe and copper pipe or tubing should never be connected directly together because of:

  - Electrolysis

  - Hydration

  - Hydrolysis

  - Hydrocorrosion

- Copper pipe is specified by its:

  - Nominal outside diameter

  - Actual outside diameter

  - Nominal inside diameter

  - Actual inside diameter

- Suppose a pump is to be removed from a plumbing system. Which of the following fittings allows the pump to be removed without disassembling the entire system?

  - Adaptor

  - Bushing

  - Union

  - Coupling

- Drain-Waste-Vent (DWV) plumbing systems may be constructed with which of the following materials? (More than one answer can be circled)

  - Copper

  - ABS

  - Cast Iron

  - Clay

- Which of the materials listed below are used to assure a leakproof joint on tapered pipe threads? (More than one answer can be circled.)

  - Teflon tape

  - PVC glue

  - Pipe joint compound with virgin Teflon

  - Pipe dope

- The primer used on PVC is to soften the plastic so the glue welds the two joints together stronger.

  - True

  - False

- Which of the following materials can be used for natural gas lines?

  - Galvanized steel

  - Copper

  - ABS

  - Black steel

- Galvanized steel pipe can be used only for cold water systems.

  - True

  - False

- PVC, ABS and PE plastic pipe can be joined to plastic fittings by means of special solvent cement (glue).

  - True

  - False

- Plastic pipe can be cut with which of the following tools:

  - Plastic pipe cutter

  - Hacksaw

  - Backsaw

  - Tubing cutter

  - All of the above

- Having an air gap between the faucet and a sink basin full of water prevents which of the following? (Include diagram of situation.)

  - Basin overflow

  - Back connections

  - Cross connections (of clean and dirty water)

  - Mixture of hot and cold water

- How many lengths of galvanized steel pipe do you need to order if you had to cut 14 faucet risers which were each 3 foot long? (SHOW ALL YOUR WORK!)